Assignment 5 : Finite State Machine

State Transition Table

| Num | Name | Lights | Input |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | waitWest | B10010100 | goWest | goWest | goWest | goWest | goWEst | goWest | goWest | goWest |
| 1 | goWest | B10100001 | goWest | goWest | waitSouth | waitSouth | wait_Walk_West | wait_Walk_West | waitSouth | waitSouth |
| 2 | waitSouth | B10100010 | goSouth | goSouth | goSouth | goSouth | goSouth | goSouth | goSouth | goSouth |
| 4 | wait_Walk_South | B10010100 | goWalk | goWalk | goWalk | goWalk | goWalk | goWalk | goWalk | goWalk |
| 5 | wait_Walk_West | B10100010 | goWalk | goWalk | goWalk | goWalk | goWalk | goWalk | goWalk | goWalk |
| 6 | goWalk | B01100100 | goWalk | button1_OFF | button1_OFF | button1_OFF | goWalk | button1_OFF | button1_OFF | button1_OFF |
| 7 | button1_OFF | B00100100 | button1_ON | button1_ON | button1_ON | button1_ON | button1_ON | button1_ON | button1_ON | button1_ON |
| 8 | button1_ON | B01100100 | button2_OFF | button2_OFF | button2_OFF | button2_OFF | button2_OFF | button2_OFF | button2_OFF | button2_OFF |
| 9 | button2_OFF | B00100100 | button2_ON | button2_ON | button2_ON | button2_ON | button2_ON | button2_ON | button2_ON | button2_ON |
| 10 | button2_ON | B01100100 | button3_OFF | button3_OFF | button3_OFF | button3_OFF | button3_OFF | button3_OFF | button3_OFF | button3_OFF |
| 11 | button3_OFF | B00100100 | button3_ON | button3_ON | button3_ON | button3_ON | button3_ON | button3_ON | button3_ON | button3_ON |
| 12 | button3_ON | B01100100 | goWalk | goWest | goSouth | goWest | goWalk | goWest | goSouth | goWest |

// West

#define LED_W_R 4

#define LED_W_Y 3

#define LED_W_G 2

#define WEST_BUTTON_PIN 11

// South

#define LED_S_R 7

#define LED_S_Y 6

#define LED_S_G 5

#define SOUTH_BUTTON_PIN 12

// Walk

#define LED_WALK_G 8

#define LED_WALK_R 9

#define WALK_BUTTON_PIN 13

// Input

#define waitWest 0

#define goWest 1

#define waitSouth 2

#define goSouth 3

#define wait_WALK_SOUTH 4

#define wait_WALK_WEST 5

#define goWalk 6

#define b1_OFF 7

#define b1_ON 8

#define b2_OFF 9

#define b2_ON 10

#define b3_OFF 11

#define b3_ON 12

กำหนด State

struct State

{

unsigned long ST_Out;

unsigned long Time;

unsigned long Next[8];

};

typedef const struct State SType;

SType FSM[13] = {

{B10010100, 400, {goWest, goWest, goWest, goWest, goWest, goWest, goWest, goWest}},                                            // waitWest

{B10100001, 2000, {goWest, goWest, waitSouth, waitSouth, wait_WALK_WEST, wait_WALK_WEST, waitSouth, waitSouth}},               // goWest

{B10100010, 400, {goSouth, goSouth, goSouth, goSouth, goSouth, goSouth, goSouth, goSouth}},                                    // waitSouth

{B10001100, 2000, {goSouth, waitWest, goSouth, waitWest, wait_WALK_SOUTH, wait_WALK_SOUTH, wait_WALK_SOUTH, wait_WALK_SOUTH}}, // goSouth

{B10010100, 400, {goWalk, goWalk, goWalk, goWalk, goWalk, goWalk, goWalk, goWalk}},                                            // wait_WALK_SOUTH

{B10100010, 400, {goWalk, goWalk, goWalk, goWalk, goWalk, goWalk, goWalk, goWalk}},                                            // wait_WALK_WEST

{B01100100, 2000, {goWalk, b1_OFF, b1_OFF, b1_OFF, goWalk, b1_OFF, b1_OFF, b1_OFF}},                                           // goWalk

{B00100100, 300, {b1_ON, b1_ON, b1_ON, b1_ON, b1_ON, b1_ON, b1_ON, b1_ON}},                                                    // b1_OFF 1

{B01100100, 300, {b2_OFF, b2_OFF, b2_OFF, b2_OFF, b2_OFF, b2_OFF, b2_OFF, b2_OFF}},                                            // b1_ON

{B00100100, 300, {b2_ON, b2_ON, b2_ON, b2_ON, b2_ON, b2_ON, b2_ON, b2_ON}},                                                    // b2_OFF 2

{B01100100, 300, {b3_OFF, b3_OFF, b3_OFF, b3_OFF, b3_OFF, b3_OFF, b3_OFF, b3_OFF}},                                            // b2_ON

{B00100100, 300, {b3_ON, b3_ON, b3_ON, b3_ON, b3_ON, b3_ON, b3_ON, b3_ON}},                                                    // b3_OFF 3

{B01100100, 300, {goWalk, goWest, goSouth, goWest, goWalk, goWest, goSouth, goWest}},                                          // b3_ON

};

unsigned long S = 0;

Setup PinLED, Button

void setup()

{

Serial.begin(9600);

pinMode(LED_W_G, OUTPUT);

pinMode(LED_W_Y, OUTPUT);

pinMode(LED_W_R, OUTPUT);

pinMode(WEST_BUTTON_PIN, INPUT_PULLUP);

pinMode(LED_S_G, OUTPUT);

pinMode(LED_S_Y, OUTPUT);

pinMode(LED_S_R, OUTPUT);

pinMode(SOUTH_BUTTON_PIN, INPUT_PULLUP);

pinMode(LED_WALK_G, OUTPUT);

pinMode(LED_WALK_R, OUTPUT);

pinMode(WALK_BUTTON_PIN, INPUT_PULLUP);

}

int West, South, Walk, input, inputPrev;

ส่วนกำหนดการแสดงผลให้โชว์LED

void loop()

{

// West

digitalWrite(LED_W_G, FSM[S].ST_Out & B00000001);

digitalWrite(LED_W_Y, FSM[S].ST_Out & B00000010);

digitalWrite(LED_W_R, FSM[S].ST_Out & B00000100);

// South

digitalWrite(LED_S_G, FSM[S].ST_Out & B00001000);

digitalWrite(LED_S_Y, FSM[S].ST_Out & B00010000);

digitalWrite(LED_S_R, FSM[S].ST_Out & B00100000);

// Walk

digitalWrite(LED_WALK_G, FSM[S].ST_Out & B01000000);

digitalWrite(LED_WALK_R, FSM[S].ST_Out & B10000000);

delay(FSM[S].Time);

West = !digitalRead(WEST_BUTTON_PIN);

South = !digitalRead(SOUTH_BUTTON_PIN);

Walk = !digitalRead(WALK_BUTTON_PIN);

input = Walk * 4 + South * 2 + West;

if(S == 6){

inputPrev = input;

}

if(S == 12){

S = FSM[S].Next[inputPrev];

}

else{

S = FSM[S].Next[input];

}

Serial.println(input);Serial.println(inputPrev);Serial.println("------------");

}